Micheal H. McCabe

CS-125 Intro to Comp Sci

Prof. Meyer

March 13, 2009

Another assignment due Monday

Write an assembly language program to input x and calculate and output 10 – x.

X:	.word				// This is our one and only variable

DECI		X,D		// Input the value of X

LOADA	10,I		// Load 10 (immediate) into the accumulator

SUBA		X,D		// Subtract X from the value in the accumulator

STOREA	X,D		// Store the result back into X

DECO		X,D		// Output the result

STOP				// End the program

Comments:

I thought I remembered an assignment using the “branch on equal” and “branch on less than” instructions in class, but I couldn’t find this in the notes posted on blackboard.

In the event that my memory is better than my computer’s, it seems like we would use a COMPA instruction to effect a comparison and depend upon flags set in the processor’s status register to effect a branch in the instructions following the comparison. This assumes that the ALU sets a flag for “equal” when the result of a comparison is zero (the Z flag on the 6502 MPU for example) and a second flag for “less than” when the result of a comparison is positive

The following code snippet uses that technique to accomplish a simple counting loop structure that counts 1-10 and exits:

X:		.word		1			// This is the counter variable

Loop:		DECO		X,D			// Output the count

ADDA		1,I			// Increment  the counter

STOREA	X,D			// Update the variable

COMPA	10,I			// Compare A with ten

BRLT		Loop,I			// If less than 10, do it again.

STOP					// Otherwise, end the program